|  | Simulation of beam with hole Date: 17 April 2024 Designer: Solidworks Study name: beam with hole Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Study Properties 3 Units 3 Material Properties 4 Loads and Fixtures 4 Connector Definitions 5 Interaction Information 5 Mesh information 5 Sensor Details 6 Resultant Forces 6 Beams 6 Study Results 7 Conclusion 9. |
| --- | --- |
| Description No Data |  |

Assumptions

Model Information Model name: beam with hole Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. Cut-Extrude1; Solid Body; Mass:75.4881 kg Volume:0.00980365 m^3 Density:7,700 kg/m^3 Weight:739.783 N; C:\Users\user\OneDrive\Documenten\Adobe\beam with hole.SLDPRT Apr 17 09:39:36 2024.

Study Properties Study name; beam with hole. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\user\OneDrive\Documenten\Adobe).

Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

Material Properties Model Reference; Properties; Components. Name:; Alloy Steel. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 6.20422e+08 N/m^2. Tensile strength:; 7.23826e+08 N/m^2. Elastic modulus:; 2.1e+11 N/m^2. Poisson's ratio:; 0.28. Mass density:; 7,700 kg/m^3. Shear modulus:; 7.9e+10 N/m^2. Thermal expansion coefficient:; 1.3e-05 /Kelvin.; SolidBody 1(Cut-Extrude1)(beam with hole). Curve Data:N/A.

Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); 0.569031; 9,999.94; -0.0825195; 9,999.94. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; 10,000 N..

Connector Definitions No Data

Interaction Information No Data

Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 21.4057 mm. Minimum element size; 7.13516 mm. Mesh Quality; High. Mesh information - Details Total Nodes; 15544. Total Elements; 9896. Maximum Aspect Ratio; 4.8319. % of elements with Aspect Ratio < 3; 99.7. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:02. Computer name:; DUSABE Kenedy.

Sensor Details No Data

Resultant Forces Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.569031; 9,999.94; -0.0825195; 9,999.94. Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.804934; -0.558834; 3.36758; 3.50725. Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

Beams No Data

Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.793e+03N/m^2 Node: 3158; 3.227e+07N/m^2 Node: 4. beam with hole-beam with hole-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 3; 7.451e-01mm Node: 9. beam with hole-beam with hole-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 2.818e-09 Element: 4303; 1.103e-04 Element: 2793. beam with hole-beam with hole-Strain-Strain1. Name:; Type. Displacement1{1}; Deformed shape. beam with hole-beam with hole-Displacement-Displacement1{1}.

Conclusion